29/04/2021

100448597

An investigation into AC Industrial Power Systems Applications Design Specifications power load flow studies using MATLAB/Simulink

6EJ528

Appendix

Table of contents

# List of figures

# List of Tables

# Introduction

# Part A – AC industrial power systems

## Three distribution winding transformers

### Theory

Mention center tap transformers and load maybe

Three winding transformers, also known as centre tap transformers or dual voltage transformers are very commonly used for distribution applications in industry and homes as it allows to split the secondary voltage into two windings, this allows for various different loads to be attached to transformer output windings. The various secondary windings can be connected either in series or parallel configurations to obtain certain voltages and currents (Tutorials, 2018).

Figure shows a diagram of a three winding distribution transformer. The primary winding is given by NP and the secondary winding is Ns=NA+NB. Eq. shows the formula for an ideal transformer

| NPNs=VPVS=ISIP | () |
| --- | --- |

Since the ratio of number of turns of the primary to the secondary is proportional to the ratio of the voltages, this means that the two voltages are VA=Vs2 and VB=VS2. For this to be possible, the common ground must be connected in between both secondary windings.

### Methodology

Figure shows the Simulink model built using a three-winding distribution transformer. This circuit is powered by a single phase and is made up of three resistive, inductive, and capacitive loads. The transformer primary is connected to a high voltage source (14,400 V rms). Two identical inductive loads (20 kW -10 kvar) are connected to the two secondaries. A third capacitive load (30 kW -20 kvar) is fed at 240 V. Windings 2 and 3 are connected in series with one another and the connection is used as the neutral wire. Initially, the circuit breaker in series with Load 2 is closed, so that the system is balanced. However, at 50ms, the circuit breaker connected to load 2 opens and the system is no longer balanced.

This circuit can represent many different applications such as a small factory, where there is a large input power, and it needs to be divided into different machines which each have different power consumptions. This is possibly a car factory where each stage of the assembly line requires different types of machines and has a certain power requirement.

### Results & Analysis

Show the 3 voltages and current and p and q

Show calculations for each one (V,I,) balanced and unbalanced (after 50ms)

Talk about the center tap transformer

## Dynamic load and programmable voltage source

### Methodology

### Results & Analysis

# Part B – Load flow studies

## Jacobi method

## Gauss-Seidal method

# Conclusions

# References

# Appendix